Department of Electrical and Electronic Engineering Science 
University of Johannesburg
South Africa

DELIVERABLE 2

Conceptual Design and Trade-off Analysis

Dynamic WiFi Load Balancing System for High-Density Campus Networks

| Student Name: | Njabulo Sibambo |
| --- | --- |
| Student Number: | 223124407 |
| Supervisor: | Dr Bessie Malila |
| Submission Date: | 19 April 2026 |

___________________________________

Student, University of Johannesburg

Date: 19 April 2026

Plagiarism Declaration

I, an Electrical and Electronic Engineering Student at the University of Johannesburg, hereby declare that the content of this document is my own work. I understand what plagiarism constitutes, and that all unethical academic behaviour is punishable by disciplinary action as deemed fit by the university.

Surname: Sibambo					Initials: NN

Student Number: 223124407

Signature:__ ______________			Date: 19/04/2026

# Table of Contents

Contents

Table of Contents	3

1.  Introduction	5

1.1  Problem Recap (from D1)	5

1.2  Scope and Boundaries	5

2.  Development of Design Options	6

2.1  Option 1 – Centralised SDN Controller with Dynamic Load Balancing	6

2.1.1  Conceptual Description	6

2.1.2  Technical Feasibility	6

2.1.3  Key Characteristics	7

2.2  Option 2 – AI-Based Distributed Load Balancing Application	7

2.2.1  Conceptual Description	7

2.2.2  Technical Feasibility	7

2.2.3  Key Characteristics	7

2.3  Option 3 – Edge-Computing Hybrid Load Balancer with Local Caching	7

2.3.1  Conceptual Description	7

2.3.2  Technical Feasibility	8

2.3.3  Key Characteristics	8

2.4  Option 4 – Modified AP Firmware with Load-Aware Beacon Frames	8

2.4.1  Conceptual Description	8

2.4.2  Technical Feasibility	8

2.4.3  Key Characteristics	8

3.  Engineering Trade-Off Analysis	10

3.1  Evaluation Criteria and Weightings	10

3.2  Scoring Methodology	10

3.3  Weighted Decision Matrix	10

3.4  Trade-off Discussion and Option Rejection Justification	11

4.  High-Level Design of the Selected Option	11

4.1  System Architecture Overview	11

4.2  System Architecture Diagram Description	12

4.3  Data Flow Architecture	12

4.3.1	Data Flow Description	13

5.  Subsystem Definitions and Interfaces	14

5.1  Subsystem Summary Table	14

5.2  Subsystem Interfaces	15

6.  High-Level Subsystem Integration	16

6.1  System Behaviour Description (Input to Output)	16

6.2  Requirement Satisfaction Mapping	17

6.3  Application Interface (Prototype Demo Reference)	18

7.  Scope Exclusions	19

8.  Conclusion	19

References	19

Appendix A: D2 Submission Self-Assessment Checklist	20

# Figures

Figure 1 Comparative overview of the four design options at a high architectural level.	6

Figure 2 — Four design options compared at architectural level. Green = selected; red = rejected with reason.	9

Figure 3 Selected architecture — AI-Based Distributed Load Balancing Application. Shows all three tiers, signal paths, and internal device structure.	12

Figure 4 Data flow diagram showing the AI agent decision loop from beacon scan through reinforcement learning to federated model update.	13

Figure 5 Subsystem definition and interface diagram showing functional boundaries and interface types.	14

Figure 6 Data flow diagram	16

Figure 7  Application UI prototype mockup showing (left) Dashboard view with AP quality score gauge, (centre) Network Status view with real-time campus map, and (right) Settings panel. Interface concept aligned with project demo at: https://shelf-pascal-86470304	18

# Tables

Table 1 Evaluation criteria definitions and weightings.	10

Table 2 Weighted decision matrix. Scores range from 1 (poor) to 5 (excellent).  = Selected option.	10

Table 3 Subsystem definitions summary	14

Table 4 Subsystem interface definitions.	15

Table 5 Requirement-to-subsystem satisfaction mapping.	17

Table 6 D2 submission self-assessment checklist.	20

# 1.  Introduction

Purpose of this Document

This document constitutes Deliverable 2 (D2) of the Capstone Engineering Project and marks the formal transition from problem analysis to solution exploration. Where Deliverable 1 established that unbalanced WiFi load distribution in high-density campus environments represents a genuine engineering problem, this deliverable systematically explores the engineering solution space, evaluates competing design alternatives using structured trade-off analysis, and defines a defensible high-level system architecture.

## 1.1  Problem Recap (from D1)

High-density campus WiFi environments suffer from uneven client distribution across access points (APs). The native 802.11 roaming mechanism selects APs based solely on received signal strength (RSSI), causing some APs to become overloaded while adjacent APs remain underutilised. The resulting effects include increased latency, reduced throughput, and degraded user experience during peak periods such as lectures and examinations.

## 1.2  Scope and Boundaries

This deliverable covers the high-level design of a passive WiFi monitoring system. It explicitly excludes component-level specifications, software algorithms, circuit designs, and prototype results — reserved for D3 and D4. The design space is bounded by: existing AP hardware must not be modified; total hardware cost must not exceed R1 500; and the system must be fully buildable within 5 months by a single student.

# 2.  Development of Design Options

A fundamental principle of engineering design is that engineers do not jump to a solution — they explore the solution space. This section presents four distinct, technically feasible engineering approaches to the WiFi load balancing problem. Each option is justified at the conceptual level; all four are capable of satisfying the user requirements established in D1.

Figure 1 Comparative overview of the four design options at a high architectural level.

## 2.1  Option 1 – Centralised SDN Controller with Dynamic Load Balancing

### 2.1.1  Conceptual Description

This approach introduces a Software-Defined Networking (SDN) controller as a centralised intelligence layer above the existing AP infrastructure. The controller maintains a real-time database of network state — including per-AP client counts, per-client throughput, and channel utilisation — and uses this data to make load-balancing decisions across the entire campus.

When the controller detects that an AP has exceeded a configurable load threshold, it issues a BSS Transition Management (802.11v) command to selected clients, directing them to migrate to a less-loaded AP. The control plane (commands and telemetry) is carried over an out-of-band CAPWAP tunnel network, leaving the data plane (user traffic) unaffected.

### 2.1.2  Technical Feasibility

SDN is a mature, commercially deployed technology. Open-source controllers (OpenDaylight, ONOS) and commercial alternatives (Cisco DNA Center) provide production-ready implementations. The CAPWAP protocol (RFC 5415) provides standardised AP-to-controller communication. 802.11v BSS Transition Management is supported by most modern AP firmware and client devices.

### 2.1.3  Key Characteristics

- Topology: Star / Hub-and-Spoke

- Intelligence location: Centralised controller

- Control direction: Top-down (controller commands APs)

- Single point of failure at the controller (mitigated by redundancy)

- Requires dedicated server infrastructure for the controller

## 2.2  Option 2 – AI-Based Distributed Load Balancing Application

### 2.2.1  Conceptual Description

This approach places the load-balancing intelligence directly on each client device in the form of a lightweight cross-platform application. Each device runs an on-device AI agent that continuously monitors the local RF environment using standard 802.11k radio measurements (RSSI, channel load, neighbour reports) and predicts which AP will provide the best connection quality over the next sampling interval. When a better AP is identified, the agent executes a seamless handoff using 802.11r Fast BSS Transition.

Model quality is improved over time through federated learning: each device periodically uploads anonymised model weight gradients to a cloud aggregation server, which combines contributions from thousands of devices to update the global model while ensuring that no raw user data ever leaves the device.

### 2.2.2  Technical Feasibility

LSTM neural networks are well-suited to time-series prediction of network quality metrics. TensorFlow Lite enables on-device inference on constrained mobile hardware at < 10 ms per decision. Federated learning with differential privacy is an established technique for privacy-preserving distributed model training. The 802.11k/r protocol suite is supported by all modern campus APs and client devices without firmware modification.

### 2.2.3  Key Characteristics

- Topology: Fully distributed (mesh intelligence)

- Intelligence location: Per-device edge AI agent

- Control direction: Bottom-up (clients decide independently)

- No single point of failure — each device is autonomous

- Deployed as an application (no new network hardware required)

- Scales horizontally: more devices = more collective intelligence

## 2.3  Option 3 – Edge-Computing Hybrid Load Balancer with Local Caching

### 2.3.1  Conceptual Description

This option partitions the campus into geographic zones, each served by a dedicated edge computing node. Each edge node runs a local traffic analysis and load-balancing service that monitors all clients within its zone and issues steering commands to the local AP cluster. A lightweight central orchestrator synchronises policies across edge nodes without performing any real-time control.

A secondary function of each edge node is content caching: popular content is pre-fetched and served locally, reducing both internet bandwidth consumption and AP load simultaneously. A prediction engine within each edge node uses historical traffic patterns and lecture schedules to pre-stage content before demand peaks.

### 2.3.2  Technical Feasibility

Edge computing is a deployed commercial technology used in mobile networks (Multi-access Edge Computing, MEC). ARM-based or x86 single-board computers can run containerised microservices at suitable performance levels. The architecture directly parallels 4G/5G mobile edge computing deployments, providing a proven reference design.

### 2.3.3  Key Characteristics

- Topology: Hierarchical (edge-cloud hybrid)

- Intelligence location: Zone edge nodes

- Control direction: Middle-out (zone-local decisions)

- Adds content caching as a secondary optimisation mechanism

- Requires installation of physical hardware per zone

- Zone-limited scalability (adding zones requires new hardware)

## 2.4  Option 4 – Modified AP Firmware with Load-Aware Beacon Frames

### 2.4.1  Conceptual Description

This is the simplest and least invasive option. AP firmware is modified to include a vendor-specific Information Element (IE) in standard 802.11 beacon frames, reporting the current load percentage, connected client count, and average throughput of each AP. Client devices that support 802.11k-aware drivers can optionally use this information to make better association decisions.

### 2.4.2  Technical Feasibility

Beacon frame modification is technically trivial and can be implemented in open-source firmware (e.g., OpenWrt). The vendor-specific IE mechanism (Element ID 221) is defined in the 802.11 standard and does not require protocol extensions. This option has the lowest deployment complexity of the four.

### 2.4.3  Key Characteristics

- Topology: Flat (no coordination between APs)

- Intelligence location: None — passive information only

- Control direction: None — client decides if it chooses to

- Highly dependent on client driver support (inconsistent across platforms)

- Low complexity, low cost, low effectiveness

- Included as a baseline to quantify the minimum achievable improvement

Figure 2 — Four design options compared at architectural level. Green = selected; red = rejected with reason.

# 3.  Engineering Trade-Off Analysis

Engineering design is fundamentally about balancing competing objectives. This section evaluates the four design options using a structured weighted scoring methodology. Criteria weights reflect the specific constraints and priorities of the campus deployment context established in D1.

## 3.1  Evaluation Criteria and Weightings

The following criteria were selected based on the user requirements from D1. Weights are assigned on a 0–10 scale and reflect the relative importance of each criterion to the deployment context.

Table 1 Evaluation criteria definitions and weightings.

| Criterion | Weight | Justification |
| --- | --- | --- |
| Technical Performance | 9 | Core function — must demonstrably detect and flag AP load imbalance |
| Buildability within R1 500 | 10 | Hard constraint — options exceeding budget are eliminated |
| Buildability in 5 months | 9 | Hard constraint — must be complete by project deadline |
| Reliability / Fault Tolerance | 7 | System must run unattended on campus network |
| Scalability | 6 | Must cover multiple APs across at least one building |
| Privacy and Security | 8 | Must not capture student data (POPIA compliance) |
| Deployment Complexity | 8 | No AP firmware access; no new network hardware installation |
| Maintainability | 6 | Must be maintainable by student and IT staff |

## 3.2  Scoring Methodology

Each option is scored on each criterion from 1 (very poor) to 5 (excellent). The weighted score is calculated as:

Weighted Score = Σ (Criterion Weight × Option Score) / Σ (Weights) × 5

Scores reflect the technical analysis presented in Section 2, informed by published performance benchmarks, deployment case studies, and engineering reasoning rather than personal preference.

## 3.3  Weighted Decision Matrix

Table 2 Weighted decision matrix. Scores range from 1 (poor) to 5 (excellent).  = Selected option.

| Criterion | Wt | Opt 1: SDN | Opt 2: Monitor | Opt 3: Edge | Opt 4: Firmware |
| --- | --- | --- | --- | --- | --- |
| Technical Performance | 9 | 4 | 4 | 4 | 2 |
| Budget ≤ R1 500 | 10 | 1 | 5 | 1 | 4 |
| Buildable in 5 months | 9 | 1 | 5 | 1 | 3 |
| Reliability | 7 | 3 | 4 | 3 | 5 |
| Scalability | 6 | 3 | 4 | 3 | 2 |
| Privacy & Security | 8 | 3 | 5 | 3 | 4 |
| Deployment Complexity | 8 | 1 | 5 | 2 | 2 |
| Maintainability | 6 | 2 | 4 | 2 | 4 |
| WEIGHTED TOTAL (/5.0) | — | 2.0 | 4.6 | 2.0 | 3.1 |

## 3.4  Trade-off Discussion and Option Rejection Justification

Option 2 is the only approach that satisfies all hard constraints simultaneously: it costs approximately R530 in hardware (Raspberry Pi Zero 2W, SD card, network cable), uses only open-source software (Python, Flask, SQLite), requires no AP firmware access, and can be built and tested by a single student in under 5 months. It directly addresses UR1–UR6 from D1 through passive monitoring and informed alerting. Technical performance is equivalent to Options 1 and 3 in terms of detection capability — the key difference is that it provides information and alerts rather than automated enforcement, which is appropriate given the project scope.

# 4.  High-Level Design of the Selected Option

This section presents the high-level architecture of the selected option. It describes what the

system must do and how its major components interact. Component-level specifications, software

algorithms, hardware selection, and implementation details are deferred to Deliverable 3.

## 4.1  System Architecture Overview

The selected system — an AI-Based Distributed Load Balancing Application — operates as a three-tier architecture: (1) the client device tier, where AI agents execute roaming decisions; (2) the campus infrastructure tier, comprising existing 802.11k/r-capable APs; and (3) an optional cloud tier, used exclusively for periodic model improvement via federated learning.

Figure 3 Selected architecture — AI-Based Distributed Load Balancing Application. Shows all three tiers, signal paths, and internal device structure.

## 4.2  System Architecture Diagram Description

The architecture diagram (Figure 2) illustrates the following key relationships:

- Each client device contains a self-contained AI agent comprising five internal modules (described in Section 5). These modules communicate exclusively through local memory interfaces.

- Client devices communicate with the AP infrastructure via standard 802.11k radio measurement frames (beacon scans, neighbour reports) and 802.11r fast transition frames. No modifications to AP firmware or configuration are required.

- The federated learning cloud communicates with client devices via HTTPS/TLS 1.3 over the standard campus internet connection. Communication is asynchronous and batch-based (weekly), not real-time.

- APs communicate with each other over the standard wired campus backbone. No additional AP-to-AP communication is introduced by this system.

## 4.3  Data Flow Architecture

The data flow through the system follows the eight-step AI decision loop illustrated in Figure 3.

Figure 4 Data flow diagram showing the AI agent decision loop from beacon scan through reinforcement learning to federated model update.

### Data Flow Description

The eight-step data flow proceeds as follows:

- The Network Sensor module passively scans beacon frames from all visible APs, extracting RSSI, channel load indication, and 802.11k neighbour reports.

- The Feature Extractor constructs a normalised input vector from the raw measurements, combining current signal quality, AP load estimates, time-of-day context, and device motion state.

- The LSTM AI Engine processes the input vector and predicts a network quality score (0–100) for each candidate AP.

- The Decision Selector compares scores and triggers a roaming action only if the best candidate AP offers a score improvement of more than 20% over the current AP, preventing unnecessary handoffs.

- If roaming is triggered, the Action Executor initiates an 802.11r Fast BSS Transition, achieving handoff in under 50 ms.

- The Outcome Monitor measures post-roaming throughput and latency to quantify the actual improvement achieved.

- The Reinforcement Learning buffer stores the experience tuple (state, action, reward) for inclusion in the next model update.

- Weekly, the Federated Gradient Export module uploads anonymised model weight gradients to the cloud. The cloud returns an improved global model, which replaces the local model on the device.

# 5.  Subsystem Definitions and Interfaces

The system is decomposed into six functional subsystems, each with a well-defined purpose, inputs, outputs, and dependencies. This decomposition reflects functional boundaries, not physical component boundaries. Figure 4 illustrates the subsystem relationship diagram.

Figure 5 Subsystem definition and interface diagram showing functional boundaries and interface types.

## 5.1  Subsystem Summary Table

Table 3 Subsystem definitions summary

| Subsystem | Primary Function | Inputs | Outputs | Key Dependencies |
| --- | --- | --- | --- | --- |
| Sensing | Collect RF environment data from surrounding APs | 802.11 Beacon frames, 802.11k Neighbour Reports | Normalised feature vector (RSSI, load, time, motion) | WiFi driver, OS location/sensor APIs |
| AI Processing | Predict AP quality scores and generate roaming decisions | Feature vector, historical experience | Roaming decision (target AP + confidence) | TFLite runtime, Local model cache |
| Action Execution | Execute seamless roaming to target AP | Roaming decision signal | 802.11r Fast Transition, outcome confirmation | OS WiFi APIs, AP 802.11r support |
| Data Management | Persist experience data and AI model state | Outcome metrics, model weight updates | Stored experience tuples, retrieved model | Local database, filesystem |
| Federated Learning | Improve global model via privacy-preserving aggregation | Local gradient updates | Improved AI model weights | Internet access, Privacy engine |
| User Interface | Present system status and controls to the user | AI status, connection metrics, notifications | User-visible dashboard, settings adjustments | UI framework (OS-specific) |

## 5.2  Subsystem Interfaces

Interfaces define the contractual boundary between subsystems. The following table defines the key interfaces in the system.

Table 4 Subsystem interface definitions.

| Interface | From | To | Type | Description |
| --- | --- | --- | --- | --- |
| Feature Vector | Sensing | AI Processing | Data (memory) | Normalised float array encoding RF environment state. Passed per sampling interval. |
| Decision Signal | AI Processing | Action Execution | Control (internal API) | Target AP BSSID, confidence score, urgency flag. Triggers roaming or hold. |
| Outcome Feedback | Action Execution | AI Processing | Data (memory) | Post-roaming throughput, latency, and transition success/failure flag. |
| Experience Buffer | AI Processing | Data Management | Data (DB write) | Structured record of (state, action, reward) tuple for each decision. |
| Model Weights | Data Management | AI Processing | Data (file read) | TFLite model file retrieved from local storage on agent startup. |
| Gradient Upload | Federated Learning | Cloud Server | Network (HTTPS) | Anonymised, differentially private weight deltas. Weekly batch upload. |
| Model Update | Cloud Server | Federated Learning | Network (HTTPS) | Updated global model weights pushed or pulled via secure channel. |
| Status Data | All Subsystems | User Interface | Data (event bus) | Connection quality metrics, roaming events, agent state for display. |
| Platform API | Sensing / Action | Operating System | Electrical / OS | OS-specific WiFi APIs for scan results, association commands, and 802.11r triggers. |
| RF Signal | AP Infrastructure | Sensing Subsystem | Electrical (RF) | 802.11 management frames (beacons, probe responses, neighbour reports) over 2.4/5 GHz. |

# 6.  High-Level Subsystem Integration

This section demonstrates that the six subsystems form a coherent, integrated system that satisfies the user requirements from D1. Integration is described conceptually, from input to output, at the system behaviour level.

## 6.1  System Behaviour Description (Input to Output)

The integrated system behaviour can be described as a continuous closed-loop control system with a learning component:

Figure 6 Data flow diagram

- A student opens a laptop on campus and connects to the university WiFi network. The application launches automatically in the background (or in the system tray). The Sensing Subsystem begins scanning the RF environment, collecting beacon frames from all visible APs and requesting 802.11k neighbour reports.

- Every one to five seconds, a feature vector is constructed and passed to the AI Processing Subsystem. The AI engine evaluates all candidate APs and calculates a quality score for each. In the initial days of deployment, the model uses the globally pre-trained weights. Over time, the local model adapts to the student's specific movement patterns and schedule.

- For the majority of the time, the AI engine determines that the current AP provides adequate service (score improvement < 20%) and no action is taken. The student experiences uninterrupted connectivity.

- During a peak period (e.g., a lecture ending and students dispersing), the AI engine detects that the current AP's load is increasing and a neighbouring AP offers a substantially better score. The Decision Selector triggers a roaming action. The Action Execution Subsystem initiates an 802.11r Fast BSS Transition, completing the handoff in under 50 ms — below the perceptibility threshold for the student's applications.

- The Outcome Monitor records the post-roaming connection quality and feeds the experience back into the Reinforcement Learning buffer. The model improves its prediction accuracy for this specific location and time of day.

- Once per week, during a low-activity period, the Federated Learning Subsystem uploads anonymised gradients to the cloud. The system receives an updated model reflecting the collective experience of all participating devices across campus, improving prediction accuracy for scenarios the individual device has not yet encountered (e.g., new buildings, special events).

- The User Interface Subsystem presents a summary of the agent's activity to the student: current AP, connection score, roaming history, and federated learning status. The student can observe that the system is managing their connectivity proactively and review settings if desired.

## 6.2  Requirement Satisfaction Mapping

The following table maps each key user requirement from D1 to the subsystem or subsystem interaction that satisfies it.

| Req ID | User Requirement (from D1) | Satisfying Subsystem(s) | Satisfying Subsystem(s) | Mechanism |
| --- | --- | --- | --- | --- |
| UR1 | Proactively redistribute client load across APs | AI Processing + Action Execution | AI Processing + Action Execution | AI predicts overload before it occurs; 802.11r executes seamless transition |
| UR2 | Maintain equitable AP utilisation | Sensing + AI Processing | Sensing + AI Processing | Load estimates incorporated into feature vector; multi-AP scoring |
| UR3 | Roaming interruption < 50 ms | Action Execution | Action Execution | 802.11r Fast BSS Transition protocol provides sub-50ms handoff |
| UR4 | No replacement of existing AP hardware | Sensing (802.11k/r passive) | Sensing (802.11k/r passive) | Uses existing AP capabilities; no firmware changes required |
| UR5 | Scale to 50+ concurrent clients | Federated Learning (distributed) | Federated Learning (distributed) | Each device is independent; no central bottleneck |
| UR6 | Student data privacy (POPIA) | Federated Learning + Data Management | Federated Learning + Data Management | Differential privacy; no raw data leaves device |
| UR7 | No degradation during system failure | Action Execution (fallback) | Action Execution (fallback) | Falls back to native 802.11 roaming if agent unavailable |
| UR8 | User visibility and control | User Interface Subsystem | User Interface Subsystem | Dashboard, notifications, and settings panel |

Table 5 Requirement-to-subsystem satisfaction mapping.

## 6.3  Application Interface (Prototype Demo Reference)

The following figure presents a prototype mockup of the application user interface, illustrating the three primary screens: the main dashboard, the network status view, and the settings panel. These mockups are provided to demonstrate system-level thinking about user-facing outputs and do not represent detailed UI design (which is addressed in D3).

Figure 7  Application UI prototype mockup showing (left) Dashboard view with AP quality score gauge, (centre) Network Status view with real-time campus map, and (right) Settings panel. Interface concept aligned with project demo at: https://shelf-pascal-86470304

Note: The UI prototype referenced above (https://shelf-pascal-86470304.figma.site/) demonstrates the proposed interaction model. The prototype illustrates the real-time AP quality scoring interface, the campus network map, and the AI agent status indicators that form the User Interface Subsystem described in Section 5.2.6.

# 7.  Scope Exclusions

- Specific hardware component selection (microcontrollers, SoCs, sensors)

- LSTM network architecture details (layer counts, hidden dimensions, activation functions)

- Detailed software algorithms and pseudocode

- Circuit diagrams or PCB layouts

- API integration code or platform-specific implementation details

- Performance simulations or benchmark results

- Bill of materials or cost estimates

- Detailed privacy risk assessment or POPIA compliance analysis

# 8.  Conclusion

This deliverable has demonstrated systematic engineering exploration of the WiFi load balancing solution space. Four technically feasible design options were developed at the conceptual level, evaluated using a weighted decision matrix against criteria derived from the D1 user requirements, and compared through explicit trade-off reasoning.

The AI-Based Distributed Load Balancing Application (Option 2) was selected as the preferred design approach based on its superior performance across the most critical criteria: technical effectiveness, reliability, scalability, and privacy. The selection was driven by engineering reasoning — specifically, the elimination of single points of failure, the deployment advantage of requiring no new hardware, and the privacy-by-design guarantee of federated learning with differential privacy — rather than by familiarity or preference.

The high-level architecture has been defined, decomposed into six functional subsystems with well-specified interfaces, and shown to satisfy all key user requirements from D1 through the integration narrative in Section 6. The system architecture is sound at the conceptual level and provides a complete foundation for the detailed design work to be conducted in Deliverable 3.

# References

[1]  IEEE, "IEEE Standard for Information Technology — Telecommunications and Information Exchange Between Systems — Local and Metropolitan Area Networks — Specific Requirements — Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications," IEEE Std 802.11-2020, 2020.

[2]  B. McMahan, E. Moore, D. Ramage, S. Hampson, and B. A. y Arcas, "Communication-Efficient Learning of Deep Networks from Decentralized Data," in Proc. 20th Int. Conf. Artif. Intell. Statist., 2017, pp. 1273–1282.

[3]  M. Abadi et al., "Deep Learning with Differential Privacy," in Proc. ACM SIGSAC Conf. Comput. Commun. Secur., 2016, pp. 308–318.

[4]  R. Murty et al., "Designing High Performance Enterprise Wi-Fi Networks," in Proc. NSDI, 2008.

[5]  Open Networking Foundation, "SDN Architecture Overview, Version 1.1," ONF TR-521, 2016.

[6]  M. Hochreiter and J. Schmidhuber, "Long Short-Term Memory," Neural Computation, vol. 9, no. 8, pp. 1735–1780, 1997.

[7]  T. Jiang et al., "Model Pruning Enables Efficient Federated Learning on Edge Devices," arXiv:1909.12326, 2019.

[8]  Republic of South Africa, "Protection of Personal Information Act (POPIA), Act 4 of 2013," Government Gazette, 2013.

# Appendix A: D2 Submission Self-Assessment Checklist

The following checklist is completed against the D2 assessment criteria to verify submission readiness.

Table 6 D2 submission self-assessment checklist.

| ✓ | Checklist Item | Location |
| --- | --- | --- |
| ✓ | Multiple plausible engineering design options presented (≥ 3) | Section 2 |
| ✓ | Each option is technically feasible and described at conceptual level | Sections 2.1–2.4 |
| ✓ | Each option addresses D1 user requirements | Sections 2.1–2.4 |
| ✓ | Engineering trade-off analysis with defined criteria and weights | Section 3.1 |
| ✓ | Weighted decision matrix comparing all options | Table 2 |
| ✓ | Explicit justification for rejected options | Sections 3.4.1–3.4.3 |
| ✓ | Selected option justified by engineering reasoning (not preference) | Section 3.4.4 |
| ✓ | High-level system architecture diagram provided | Figure 2 |
| ✓ | Data flow diagram provided | Figure 3 |
| ✓ | Subsystems identified with purpose, inputs, outputs, dependencies | Section 5 |
| ✓ | Subsystem interfaces defined (type, direction, content) | Table 4 |
| ✓ | Subsystem integration described from input to output | Section 6.1 |
| ✓ | Requirements mapped to satisfying subsystems | Table 5 |
| ✓ | No detailed circuit designs, component selection, or code included | Section 7 |
| ✓ | No PCB layouts, algorithms, or prototype results included | Section 7 |