DEPARTMENT OF ELECTRICAL AND ELECTRONIC ENGINEERING SCIENCE

University of Johannesburg  |  South Africa

COMPLEX ENGINEERING PROBLEM

Description, High level Design and Detailed Design Report

(D1 + D2 + D3 Combined)

Intelligent WiFi Congestion Monitoring and Recommendation System

| Field | Detail |
| --- | --- |
| Student Name | Njabulo Sibambo |
| Student Number | 223124407 |
| Supervisor | Dr Bessie Malila |
| Department | Electrical and Electronic Engineering Science |
| Institution | University of Johannesburg |
| Submission Date | June 2026 |
| Deliverables | D1 – Problem Description ¦ D2 – Design ¦ D3 – Detailed Design |

PLAGIARISM DECLARATION

I, an Electrical and Electronic Engineering student at the University of Johannesburg, hereby declare that the content of this document is my own work. I understand what plagiarism constitutes, and that all unethical academic behaviour is punishable by disciplinary action as deemed fit by the university.

Surname: Sibambo          Initials: NN          Student Number: 223124407

Signature: _SIBAMBO_____________          Date: 2026/05/21

Table of Contents

1. Problem Description	7

1.1 System Boundary Definition	7

1.2 Stakeholders Affected	8

2. Argument That This Is a Complex Engineering Problem	9

2.1 Systems-Level Interactions	9

2.2 Conflicting Technical Constraints	9

2.3 Requirement for First-Principles Analysis	10

Shannon-Hartley Theorem	10

M/M/1 Queueing Model	10

2.4 Absence of Standard Solutions	11

3. User Requirements	11

4. Engineering Fundamentals and Scientific Underpinning	12

5. Resource Use, Environmental Impact and Lifecycle Considerations	12

6. Five Engineering Solution Alternatives	16

6.1 Alternative 1 – Access Point Hardware Upgrade	16

6.2 Alternative 2 – Enterprise Wireless LAN Controller (WLC)	16

6.3 Alternative 3 – AP-Side Client Load Balancing Firmware	16

6.4 Alternative 4 – 802.11k/v-Based Assisted Roaming	17

6.5 Alternative 5 – On-Device Intelligent Monitoring and Recommendation System (SELECTED)	17

7. Selected Solution: Detailed System Design	17

7.1 Core Project Concept	18

7.2 System Architecture – Five Subsystems	18

8. System Data Flow	18

9. Intelligent Scoring Algorithm	20

9.1 Scoring Formula	20

9.2 AP Classification Thresholds	21

10. Dashboard User Interface Design	21

11. Multi-Device Support Architecture	22

12. System Requirements	23

13. Testing Plan and Validation Strategy	23

14. Known Limitations	24

15. Introduction and Scope of D3	27

16. Refined System Architecture	27

16.1 Module File Structure	27

16.2 Interface Definitions	28

17. Sensing Subsystem – Detailed Design	29

17.1 OS Command and Parsed Fields	29

17.2 RSSI Derivation	29

17.3 APRecord Data Structure	29

17.4 Hardware Parameters	30

18. Network Testing Subsystem – Detailed Design	30

18.1 Ping Command	30

18.2 Derived Performance Metrics	31

18.3 Stability Score Derivation	31

18.4 PingResult Data Structure	31

19. Intelligent Scoring Subsystem – Detailed Design	31

19.1 Normalisation Specifications	31

Signal Strength Normalisation	32

Latency Normalisation	32

Packet Loss Normalisation	32

Stability Normalisation	32

19.2 Recommendation Decision Logic	32

19.3 ScoredAP Data Structure	32

20. Data Storage Subsystem – Detailed Design	32

20.1 CSV Schema	33

20.2 File Naming and Write Operation	33

20.3 Storage Capacity Calculation	34

21. Engineering Calculations	34

21.1 Shannon-Hartley Per-User Throughput	34

21.2 M/M/1 Queueing Latency Prediction	34

21.3 Measurement Accuracy Analysis	35

21.4 CPU and Memory Footprint	35

22. System Integration Design	36

22.1 Configuration Parameters	36

22.2 End-to-End Worked Example	36

Input Measurements	37

Step 1: Normalisation for AP-01 (connected)	37

Step 2: Score Calculation for AP-01	37

Step 3: AP-02 and AP-03 (not connected – signal score only, other sub-scores default to 50)	37

Step 4: Recommendation Decision	37

23. User Interface Subsystem – Detailed Design	38

23.1 Implementation: Streamlit	38

23.2 Streamlit Component Mapping	38

23.3 Recommendation Message Templates	38

24. Unit and Integration Testing Plan	39

24.1 Unit Tests	39

24.2 Integration Tests	39

25. Known Limitations and Future Work	40

25.1 Known Limitations	40

25.2 Future Work	40

References	41

PART D1

Complex Engineering Problem

Description and Overview

1. Problem Description

Wireless Local Area Networks (WLANs) allow devices such as laptops, smartphones, and tablets to connect to the internet using radio frequency (RF) signals, eliminating the need for physical cabling infrastructure. In university environments these networks constitute critical infrastructure that supports all teaching and learning activities, including programming laboratories, software installation sessions, and online examination platforms.

During laboratory sessions it is common for many students to simultaneously download large software packages, stream instructional videos, or access online assessment platforms. When this occurs inside a lecture hall or computer laboratory, dozens of devices begin requesting large volumes of data at the same time. Each device automatically connects to the wireless access point (AP) that provides the strongest Received Signal Strength Indicator (RSSI), which is the default behaviour of all standard IEEE 802.11 WiFi association algorithms [4].

This signal-strength-first association behaviour consistently produces an uneven distribution of users across the available APs in the vicinity. One AP may accumulate twenty or more connected devices while a physically nearby AP remains nearly unused. The overloaded AP is forced to share its fixed radio channel bandwidth among all connected users. According to the Shannon-Hartley Theorem, the maximum channel capacity is physically bounded by available bandwidth and signal-to-noise ratio and cannot be exceeded [2]. When this bounded capacity is divided among many simultaneous users, each individual user receives a much smaller fraction, resulting in severely degraded download speeds and long waiting times.

The engineering problem is formally defined as: the inefficient and unbalanced distribution of wireless network traffic among multiple access points in a high-density indoor environment, resulting in severe congestion and significantly degraded network performance for all users connected to overloaded access points [1, 4].

Figure 1 – WiFi Congestion in a High-Density Environment: AP 1 is overloaded while AP 2 has spare capacity [4]

## 1.1 System Boundary Definition

The system boundary defines which elements are included in the engineering analysis and which are excluded from the design scope. Table 1 presents this boundary.

| System Element | Included | Reason |
| --- | --- | --- |
| Student laptops and smartphones | Yes | Primary sources of network traffic in the high-density environment |
| Wireless access points (APs) | Yes | Provide wireless communication services to all connected devices |
| RF communication channel (2.4/5 GHz) | Yes | Physical medium through which WiFi signals propagate [5, 6] |
| Building environment (walls, layout) | Yes | Directly influences signal attenuation, reflection, and interference patterns [5] |
| On-device monitoring software | Yes | Core system under design – scans, scores, and displays AP quality |
| External internet backbone | No | Congestion occurs at the local AP level, not the ISP or backbone |
| Remote servers hosting downloads | No | Not responsible for the local traffic imbalance between APs |

Table 1 – System Boundary Definition

Figure 2 – System Boundary: Included components (inner box) and excluded components (outer, dashed)

## 1.2 Stakeholders Affected

Multiple stakeholders are directly and indirectly affected by WiFi congestion in a university environment, as described in Table 2.

| Stakeholder | Impact of WiFi Congestion |
| --- | --- |
| Students | Delayed software downloads, failed exam submissions, and interrupted learning during laboratory sessions |
| Lecturers | Reduced instructional time due to troubleshooting and extended wait for student downloads to complete |
| IT Administrators | Increased network troubleshooting workload, system monitoring demand, and rising user complaints |
| University Management | Financial pressure from infrastructure upgrade costs; reputational impact from poor service quality |

Table 2 – Stakeholders and Their Impacts

# 2. Argument That This Is a Complex Engineering Problem

According to the Engineering Council of South Africa (ECSA), a complex engineering problem is one that involves several interacting systems, presents conflicting engineering constraints, and requires analysis using engineering fundamentals such as electromagnetic theory, information theory, and mathematical modelling rather than simple configuration adjustments or standard procedures [1].

This section demonstrates that the WiFi congestion problem satisfies all ECSA criteria for a complex engineering problem.

## 2.1 Systems-Level Interactions

The WiFi network is not a single component but a collection of interacting subsystems. The overall system behaviour depends on dynamic interactions between these subsystems, which cannot be predicted or optimised by considering any one component in isolation [5, 6].

| Subsystem | Role | Engineering Discipline |
| --- | --- | --- |
| User devices | Generate and receive network traffic | Computer networking, protocol engineering |
| Access points (APs) | Distribute wireless bandwidth to connected users | Wireless communications, RF engineering |
| RF channels (2.4/5 GHz) | Carry modulated signals between devices and APs | Electromagnetic theory, information theory [2] |
| Indoor building environment | Causes signal attenuation, reflection, and multipath fading | Electromagnetics, propagation modelling [5, 6] |
| Network protocols (TCP/IP, 802.11) | Control packet routing, acknowledgment, and retransmission | Networking theory, queueing theory [3, 4] |

Table 3 – Interacting Subsystems and Their Engineering Disciplines

## 2.2 Conflicting Technical Constraints

Wireless network design must simultaneously balance several competing engineering objectives. Because these objectives conflict with each other, the system cannot be optimised using any single simple strategy [1].

| Objective | Conflict | Trade-Off |
| --- | --- | --- |
| Maximum data throughput | May reduce fairness between individual users | Speed vs. equity |
| Strong signal connection (RSSI) | May cause device to overload a specific AP | Signal strength vs. load balance [4] |
| Balanced traffic distribution | Requires temporary disconnection when switching APs | Balance vs. connection stability |
| Stable wireless connections | Prevents the system from redistributing traffic dynamically | Stability vs. dynamic optimisation |
| Low device energy consumption | Limits frequency of active scanning and monitoring | Energy efficiency vs. monitoring accuracy |

Table 4 – Conflicting Engineering Constraints

## 2.3 Requirement for First-Principles Analysis

Analysis of the WiFi congestion problem requires direct application of fundamental engineering laws and mathematical models from multiple disciplines [2, 3, 5].

### Shannon-Hartley Theorem

The Shannon-Hartley Theorem defines the theoretical maximum data rate achievable on a communication channel in the presence of noise [2]:

C = B · log₂(1 + SNR)

where C = channel capacity (bps), B = channel bandwidth (Hz), SNR = signal-to-noise ratio (linear)

This law proves that the capacity of a wireless channel is physically bounded [2]. When many users share the same AP channel simultaneously, the available capacity must be divided among them, which is the fundamental cause of congestion.

Figure 3 – Shannon-Hartley Model: Per-user throughput decreases as the number of users on one AP increases [2]

### M/M/1 Queueing Model

The M/M/1 queueing model quantifies the relationship between traffic load, waiting time, and packet delay at an access point [3]:

W = 1 / (μ − λ)

where W = mean packet waiting time, μ = AP service rate (packets/s), λ = total arrival rate (packets/s)

As the traffic load λ approaches the service rate μ, the waiting time W rises non-linearly toward infinity. This explains why small increases in user count near saturation cause disproportionately large increases in latency [3].

Figure 4 – M/M/1 Queueing Model: Mean packet delay rises exponentially as AP load approaches saturation [3]

## 2.4 Absence of Standard Solutions

Standard WiFi association algorithms defined by the IEEE 802.11 specification rely exclusively on signal strength (RSSI) to select access points [4]. These algorithms do not account for the current load on each AP or the actual congestion level. As a direct result, devices may connect to and remain connected to an already severely overloaded AP, even when neighbouring APs with significant available capacity are physically close. This represents a fundamental limitation of existing standard procedures, confirming that the problem requires engineering analysis beyond routine configuration methods [1].

# 3. User Requirements

User requirements describe the expected outcomes and performance characteristics of the system from the perspective of the stakeholders. These requirements do not prescribe specific design solutions but define measurable performance targets that any acceptable solution must satisfy.

Table 5 – User Requirements

| ID | Requirement | Measurement Method | Priority |
| --- | --- | --- | --- |
| UR1 | Support a large number of simultaneous users in one location | Number of concurrently connected devices | High |
| UR2 | Maintain acceptable download speed during peak usage periods | Average download speed in Mbps | High |
| UR3 | Ensure balanced distribution of users across available APs | Standard deviation of users per AP | High |
| UR4 | Maintain stable wireless connections for all users | Connection drop rate per session | Medium |
| UR5 | Operate within existing campus infrastructure without hardware changes | Compatibility verification – no admin access needed | High |
| UR6 | Minimise energy consumption of user devices during monitoring | Device battery usage during operation | Low |
| UR7 | Provide real-time visibility of AP congestion status to the user | Dashboard refresh rate and responsiveness | High |
| UR8 | Protect user privacy – all data must be processed and stored locally | Data transmission audit – no external uploads | High |

# 4. Engineering Fundamentals and Scientific Underpinning

Several distinct engineering and mathematical disciplines are required to fully understand, model, and address the WiFi congestion problem. Each discipline contributes specific analytical tools and theoretical frameworks.

| Engineering Field | Application to This Problem | Key Principle or Reference |
| --- | --- | --- |
| Electromagnetic Theory | Explains how radio signals propagate, attenuate through walls, and create interference | Free-space path loss equation [5, 6] |
| Wireless Communication Theory | Determines practical data transmission limits under real-world RF conditions | Shannon-Hartley: C = B·log₂(1+SNR) [2] |
| Information Theory | Defines the maximum theoretical channel capacity and the effect of noise | Shannon entropy and channel capacity bounds [2] |
| Computer Networking | Explains packet routing, congestion control, and TCP behaviour under load | TCP/IP congestion window, 802.11 MAC [4] |
| Queueing Theory | Models the relationship between traffic load, waiting time, and packet delay at each AP | M/M/1 queue: W = 1/(μ − λ) [3] |
| Graph Theory | Represents the network topology and relationships between devices, APs, and links | Weighted graph models, shortest path |
| Statistics and Data Science | Used in the scoring model to normalise raw metrics and weight multiple indicators | Weighted scoring, statistical normalisation |

Table 6 – Engineering Theory and Application

# 5. Resource Use, Environmental Impact and Lifecycle Considerations

Engineering solutions must always be evaluated for their resource usage, environmental impact, and long-term sustainability [1]. The design of the monitoring system explicitly addresses these considerations.

| Constraint | Impact | Design Response |
| --- | --- | --- |
| Hardware cost | Adding new APs requires significant capital investment | System runs on existing user devices – no new hardware required |
| Energy consumption | Congestion causes repeated TCP retransmissions, wasting device energy [8] | Better AP selection reduces retransmissions and conserves battery life |
| Maintenance complexity | Advanced infrastructure systems require specialist network technicians | On-device software is simple to update and requires no specialist support |
| Scalability | Infrastructure must support future growth in device numbers | Software scales automatically to any number of devices without modification |
| Equipment lifespan | Existing AP infrastructure should remain useful for many years | Better traffic distribution extends the effective life of current APs |

Table 7 – Resource and Lifecycle Constraints

PART D2

Complex Engineering Problem

System Design

6. Five Engineering Solution Alternatives

Before selecting a final design approach, five distinct engineering solution alternatives were identified and evaluated. The five alternatives presented here are closely related options that all address the WiFi congestion problem from a wireless network perspective. Each alternative was assessed against four key engineering criteria: feasibility within project scope, infrastructure change required, estimated cost, and expected effectiveness. The five alternatives and the multi-criteria evaluation are shown below.

Figure 5 – Five Engineering Alternatives: Multi-Criteria Comparison Matrix (Solution 5 selected)

## 6.1 Alternative 1 – Access Point Hardware Upgrade

This alternative proposes replacing existing AP hardware with newer, higher-specification 802.11ax (Wi-Fi 6) access points that support MU-MIMO (Multi-User Multiple Input Multiple Output) technology. Wi-Fi 6 APs can serve multiple devices simultaneously on distinct spatial streams, reducing effective congestion per AP. However, this requires procurement, physical installation, electrical cabling, and configuration by IT staff with full administrative access. The capital expenditure for a single lab of 20 APs ranges from R80,000 to R200,000. This alternative is not feasible within the project scope and budget [4].

## 6.2 Alternative 2 – Enterprise Wireless LAN Controller (WLC)

A centralised Wireless LAN Controller (WLC) such as a Cisco Catalyst 9800 or Aruba Mobility Controller monitors all AP loads in real time and enforces client association policies. The WLC can issue disassociation frames to clients connected to overloaded APs and redirect them to less-loaded alternatives. While this is the most effective infrastructure-based solution, it requires enterprise-grade hardware costing from R150,000, complex integration with the campus network switch fabric, and exclusive administrative access to all APs on the campus LAN. This alternative is therefore excluded due to infrastructure and budget constraints [4].

## 6.3 Alternative 3 – AP-Side Client Load Balancing Firmware

Several AP firmware implementations (including OpenWRT and some vendor firmware) support proprietary client load-balancing features. These features allow individual APs to detect when a client's RSSI exceeds a threshold, and to defer association responses, effectively steering the client toward a less-loaded AP. This approach requires firmware modification or replacement on every campus AP, which is not possible without administrative access to the AP management system. Additionally, firmware modifications carry warranty and compliance risks for the university's network infrastructure [4].

## 6.4 Alternative 4 – 802.11k/v-Based Assisted Roaming

The IEEE 802.11k (Radio Resource Management) and 802.11v (BSS Transition Management) protocol extensions allow APs to broadcast neighbour reports and send BSS Transition Management Request frames to clients, suggesting or requesting a roam to a better AP. This is a standards-compliant mechanism that preserves open roaming without forcing disconnection. However, both the campus APs and client devices must support 802.11k/v, which requires verification and often firmware updates on the AP side. Campus network administrators must also enable these features in the wireless controller configuration. This alternative is therefore not feasible without infrastructure access [4].

## 6.5 Alternative 5 – On-Device Intelligent Monitoring and Recommendation System (SELECTED)

The selected solution runs entirely on the user's own device. It continuously scans nearby APs using operating system commands, measures real-world performance metrics including signal strength, latency, and packet loss, calculates a quality score for each AP, and presents this information through a real-time colour-coded dashboard. The system recommends the best available AP to the user without requiring any changes to the network infrastructure.

This alternative was selected because it requires no hardware procurement, no infrastructure changes, no administrative access, and no additional cost. It operates entirely within the engineering constraints of a student project while delivering a measurable and practical impact on the user experience. It also satisfies all eight user requirements identified in Section 3.

Figure 6 – Engineering Constraint Trade-Off Analysis: Solution 5 dominates across key dimensions

# 7. Selected Solution: Detailed System Design

## 7.1 Core Project Concept

The Multi-Device Intelligent WiFi Congestion Monitoring and Recommendation System is an on-device software tool that helps users identify and connect to the best available wireless access point in high-density environments. The system performs all processing locally on the user's device and does not modify or interact with the network infrastructure in any way.

The system operates on three device categories: laptops (primary prototype implemented on Windows 10/11), smartphones, and tablets. On each device the system performs three core functions: it observes WiFi conditions by scanning nearby access points, analyses connection quality by measuring real-world performance metrics, and recommends better connectivity options to the user through a visual dashboard.

Figure 7 – Integrated System Architecture: Five subsystems process WiFi data in a continuous pipeline

## 7.2 System Architecture – Five Subsystems

The system is structured as five clearly defined subsystems, each responsible for a distinct function. The subsystems interact in a linear pipeline with a data storage feedback loop.

| Subsystem | Function | Input | Output |
| --- | --- | --- | --- |
| ① Sensing | Detect and list all nearby WiFi APs | RF signals from environment | List of APs with SSID, BSSID, RSSI, channel |
| ② Network Testing | Measure current connection quality via ping | Active network connection | Latency (ms) and packet loss (%) |
| ③ Scoring | Calculate a quality score for each AP using weighted formula | Normalised RSSI, latency, packet loss, stability | Score (0–100) and status classification |
| ④ Data Storage | Store all measurements with timestamps | Scores and raw measurements | Historical data in CSV file |
| ⑤ User Interface | Display results and recommendations | Scores and status classifications | Colour-coded dashboard and recommendation message |

Table 8 – Five Subsystems: Functions, Inputs and Outputs

# 8. System Data Flow

The following data flow describes the complete step-by-step operation of the system from startup to continuous real-time monitoring. The system repeats this process every 10 seconds to provide up-to-date information.

Figure 8 – Step-by-Step Data Flow Diagram: continuous monitoring loop from scan to store

| Step | Action | Technical Detail |
| --- | --- | --- |
| 1 | Launch application | User opens Python application; Streamlit dashboard loads at localhost:8501; monitoring loop initialises |
| 2 | WiFi scan | Device executes: netsh wlan show networks mode=bssid (Windows); scans all visible APs in RF vicinity |
| 3 | Identify APs | Each network uniquely identified using BSSID (MAC address); same-SSID networks treated as separate APs |
| 4 | Performance test | Ping test to default gateway: ping -n 10 -w 2000 <gateway_ip>; measures latency and packet loss |
| 5 | Normalise metrics | Raw values converted to 0–100 scale; RSSI of −30 dBm → 100; −90 dBm → 0; latency <20 ms → 100; >200 ms → 0 |
| 6 | Calculate score | Score = 0.35×Signal + 0.30×Latency + 0.20×PacketLoss + 0.15×Stability |
| 7 | Classify AP | Score ≥ 75 → GOOD (green); Score 50–74 → MODERATE (orange); Score < 50 → CONGESTED (red) |
| 8 | Display dashboard | Streamlit renders colour-coded AP table, signal bars, latency trend, current AP status |
| 9 | Recommend | If ΔScore ≥ 20: display message: 'Better connection available: [AP name]. Move toward that direction.' |
| 10 | User acts | User may move location, manually reconnect to recommended AP, or remain; system does not force action |
| 11 | Store data | All measurements appended to wifi_log_YYYYMMDD_HHMMSS.csv with ISO-8601 timestamp |
| 12 | Loop | time.sleep(10 seconds) → return to Step 2; continuous real-time monitoring throughout session |

Table 9 – Step-by-Step System Operation

# 9. Intelligent Scoring Algorithm

The core intelligence of the system is the weighted scoring algorithm. This algorithm converts multiple raw WiFi measurements into a single, easy-to-interpret quality score for each access point. The scoring approach is computationally lightweight, transparent, and explainable.

Figure 9 – Intelligent Scoring Algorithm: four normalised metrics combined with weights to produce a classified score

## 9.1 Scoring Formula

Score = 0.35 × S_signal + 0.30 × S_latency + 0.20 × S_loss + 0.15 × S_stability

| Metric | Weight | Normalisation | Justification |
| --- | --- | --- | --- |
| Signal Strength (RSSI) | 35% | −30 dBm → 100; −90 dBm → 0 | Most widely available metric; directly indicates link quality; accounts for over-reliance on RSSI by current algorithms [4] |
| Latency (ms) | 30% | <20 ms → 100; >200 ms → 0 | High latency directly degrades user experience for downloads, browsing, and online assessments [3] |
| Packet Loss (%) | 20% | 0% → 100; ≥20% → 0 | Packet loss triggers TCP retransmission and window collapse; even 5% loss can halve effective throughput [8] |
| Stability (variance) | 15% | Low variance → 100; high → 0 | A stable connection is preferable to a variable one even at the same mean quality; indicates queue stability [3] |

Table 10 – Scoring Weights, Normalisation and Justification

## 9.2 AP Classification Thresholds

| Status | Score Range | Meaning | Recommended Action |
| --- | --- | --- | --- |
| GOOD | 75 – 100 | Excellent connection, low congestion | Stay connected – best available option |
| MODERATE | 50 – 74 | Acceptable but monitor for degradation | Monitor; consider moving if better AP is visible |
| CONGESTED | 0 – 49 | Significant congestion, poor performance | Move location or manually reconnect to a GOOD AP |

Table 11 – AP Classification Thresholds

# 10. Dashboard User Interface Design

The user interface is the primary interaction point between the system and the user. It is designed to present complex network performance information in a simple, clear, and actionable format using colour-coded indicators so that users can understand AP status without any technical knowledge.

Figure 10 – Dashboard UI Mockup: AP table, status indicators, recommendation box, and latency trend graph

| Component | Content | Purpose |
| --- | --- | --- |
| Status bar | Current AP name, score, and status badge | Immediate overview of current connection quality |
| AP comparison table | All detected APs with RSSI, latency, packet loss, score, and status | Comparison of all available connection options |
| Colour indicators | Green / Orange / Red per AP row | Instant visual communication without numerical understanding required |
| Recommendation box | Best available AP name and directional guidance message | Actionable advice for the user to improve their connection |
| Signal bars | Visual bar chart of RSSI levels for all detected APs | Familiar WiFi bar representation for non-technical users |
| Latency trend graph | Time-series graph of latency over last 60 seconds | Shows whether connection is improving or degrading over time |
| Timestamp footer | Time of most recent scan | Confirms data is current and monitoring loop is running |

Table 12 – Dashboard Components and Their Purpose

# 11. Multi-Device Support Architecture

The system is designed to support three device types. The same core architecture, scoring algorithm, and dashboard design apply across all devices. The only component that differs between platforms is the WiFi sensing method, as different operating systems provide different levels of access to WiFi scanning data.

| Device | Sensing Method | Implementation Stack | Status |
| --- | --- | --- | --- |
| Laptop (Windows 10/11) | netsh wlan show networks mode=bssid | Python 3.11, Streamlit, Pandas, CSV | PRIMARY PROTOTYPE |
| Smartphone (Android) | Android WifiManager API | Android app (Java/Kotlin or Flutter) | Future Implementation |
| Tablet | Same as smartphone platform API | Same codebase, optimised for larger screen | Future Implementation |

Table 13 – Device Support Matrix

# 12. System Requirements

System requirements define the specific technical behaviours and capabilities the software must deliver to satisfy the user requirements presented in Section 3. These are derived directly from the user requirements.

| ID | System Requirement | Linked UR |
| --- | --- | --- |
| SR1 | The system shall scan all visible WiFi APs and return a list within 5 seconds | UR1, UR7 |
| SR2 | The system shall uniquely identify each AP using its BSSID | UR3 |
| SR3 | The system shall measure network latency by performing a 10-packet ICMP ping test to the default gateway | UR2, UR7 |
| SR4 | The system shall calculate packet loss percentage from the ping test results | UR2 |
| SR5 | The system shall compute a quality score (0–100) for each AP using the defined weighted formula | UR2, UR3 |
| SR6 | The system shall classify each AP as GOOD, MODERATE, or CONGESTED based on defined thresholds | UR2, UR7 |
| SR7 | The system shall display a real-time colour-coded dashboard refreshed every 10 seconds | UR7 |
| SR8 | The system shall display a recommendation message when a better AP (ΔScore ≥ 20) is available | UR2, UR3 |
| SR9 | The system shall store all measurements and scores locally in a CSV file with ISO-8601 timestamps | UR8 |
| SR10 | The system shall not transmit any data to external servers – all processing is local | UR5, UR8 |

Table 14 – System Requirements Traced to User Requirements

# 13. Testing Plan and Validation Strategy

The system will be tested in a real-world environment that simulates the high-density university laboratory scenario. Testing is conducted in four phases corresponding to different density and mobility conditions.

| Phase | Scenario | Measurement | Expected Outcome |
| --- | --- | --- | --- |
| Phase 1 | Low user density (1–5 devices on one AP) | Score, latency < 30 ms, packet loss < 1% | All APs show GOOD status; no recommendation issued |
| Phase 2 | High user density (15+ devices on one AP) | Score drop, latency > 100 ms | Dashboard shows CONGESTED; recommendation message fires |
| Phase 3 | User moves across room to a better AP | Score change per cycle | Score improves progressively; dashboard updates in real-time |
| Phase 4 | Multiple APs simultaneously visible | AP ranking accuracy | Best AP correctly highlighted; recommendation accurate |

Table 15 – D2 Testing Plan

# 14. Known Limitations

The following limitations are recognised as inherent constraints of the current design and are documented for transparency and academic integrity.

- The system cannot determine the exact physical location of each access point within the building.

- The system cannot force a device to disconnect from one AP and reconnect to another – this decision remains with the operating system.

- Hidden network load information (the number of users connected to each AP) is not accessible without infrastructure-level access.

- RSSI values are affected by physical obstacles, interference from other devices, and temporary RF reflections, which may introduce measurement variability of ±3–5 dBm [5, 6].

- Mobile operating systems (particularly iOS) impose restrictions on third-party WiFi scanning APIs, which limits sensing capability on those platforms.

- The weighted scoring formula uses fixed weights selected based on engineering judgement; these weights may be further optimised through real-world testing and machine learning in future work.

PART D3

Complex Engineering Problem

Detailed Design

# 15. Introduction and Scope of D3

Deliverable D2 established the conceptual architecture of the system and selected the on-device monitoring approach as the most feasible engineering solution. Deliverable D3 now converts that conceptual design into a complete, technically implementable specification. This section answers the question: how exactly does each subsystem work?

D3 provides component-level software design, algorithm pseudocode, data structure definitions, interface specifications, engineering calculations, and a full system integration model. The document is structured such that any qualified engineer could implement the system from this specification without further design decisions.

| D2 (Conceptual) | D3 (Technical) |
| --- | --- |
| High-level block diagram | Module-level software architecture with file names |
| Subsystem descriptions | Data structures, function signatures, pseudocode |
| Formula stated | Formula derived, bounds justified, normalisation calculated |
| CSV storage mentioned | 18-column schema defined, write operation specified, capacity calculated |
| Dashboard described | Component-to-Streamlit-element mapping, refresh mechanism, message templates |
| Testing phases listed | Unit tests with quantitative pass criteria |

Table 16 – D2 vs D3 Design Depth Comparison

# 16. Refined System Architecture

## 16.1 Module File Structure

The system is implemented as a Python 3.11 application running on Windows 10/11. It is structured as six independent modules that communicate through well-defined in-memory data structures. All processing occurs locally on the user device with no external network communication between modules [10].

Figure 11 – D3 Software Module Architecture: six Python modules with defined interfaces

| Module File | Subsystem | Responsibility |
| --- | --- | --- |
| scanner.py | Sensing | Executes netsh wlan show networks mode=bssid and parses raw output into APRecord dataclass instances |
| tester.py | Network Testing | Executes ping -n 10 -w 2000 <gateway> and computes latency, packet loss, and RTT variance statistics |
| scorer.py | Scoring | Normalises raw metrics and applies weighted scoring formula; implements recommendation decision logic |
| storage.py | Data Storage | Appends ScoredAP records to a timestamped CSV file; validates schema on first write |
| dashboard.py | User Interface | Renders real-time Streamlit dashboard at localhost:8501; reads from Streamlit session state |
| main.py | Orchestrator | Controls the main monitoring loop; coordinates data flow between all modules every 10 seconds |
| config.py | Configuration | Centralises all tunable parameters: weights, thresholds, scan interval, normalisation bounds, file paths |

Table 17 – Module File Structure and Responsibilities

## 16.2 Interface Definitions

| Interface | From | To | Data Type | Format |
| --- | --- | --- | --- | --- |
| I1 – AP List | scanner.py | scorer.py + storage.py | List[APRecord] | Python dataclass list |
| I2 – Ping Result | tester.py | scorer.py + storage.py | PingResult | Python dataclass |
| I3 – Scored APs | scorer.py | dashboard.py + storage.py | List[ScoredAP] | Python dataclass list |
| I4 – Config Params | config.py | All modules | Constants | Python module import |
| I5 – CSV Row | storage.py | Filesystem (./logs/) | String | Comma-separated UTF-8 |

Table 18 – System Interface Definitions

# 17. Sensing Subsystem – Detailed Design

## 17.1 OS Command and Parsed Fields

The sensing subsystem invokes the following Windows OS command via Python's subprocess module to retrieve WiFi scan data [7]:

import subprocess

result = subprocess.run(

['netsh', 'wlan', 'show', 'networks', 'mode=bssid'],

capture_output=True, text=True, encoding='utf-8'

)

raw_output = result.stdout

The raw multi-line text output is parsed using regular expressions to extract the following fields:

| Field | netsh Output Line | Data Type | Example |
| --- | --- | --- | --- |
| SSID | SSID : Eduroam | String | Eduroam |
| BSSID | BSSID 1 : AA:11:22:33:44:55 | String | AA:11:22:33:44:55 |
| Signal (%) | Signal : 72% | Integer | 72 |
| Channel | Channel : 6 | Integer | 6 |
| Radio type | Radio type : 802.11n | String | 802.11n (2.4 GHz) |
| RSSI (dBm) | Derived from Signal% | Float | −64.0 |

Table 19 – Sensing Subsystem Parsed Fields

## 17.2 RSSI Derivation

The Windows netsh utility reports signal quality as a percentage (0–100) rather than directly in dBm units. The RSSI in dBm is derived using the standard Windows NDIS WLAN API conversion formula [7]:

RSSI (dBm)  =  (Signal% / 2)  −  100

Validation examples:

- Signal% = 100  →  RSSI = (100/2) − 100 = −50 dBm  (excellent signal, very close to AP)

- Signal% = 72   →  RSSI = (72/2) − 100  = −64 dBm  (good signal, typical indoor range)

- Signal% = 0    →  RSSI = (0/2) − 100   = −100 dBm (no signal, hardware noise floor)

## 17.3 APRecord Data Structure

from dataclasses import dataclass

@dataclass

class APRecord:

ssid:        str    # Network name (e.g. 'Eduroam')

bssid:       str    # Unique AP MAC address (e.g. 'AA:11:22:33:44:55')

signal_pct:  int    # Windows signal quality 0–100

rssi_dbm:    float  # Derived: (signal_pct / 2) − 100

channel:     int    # WiFi channel number (1–14 for 2.4 GHz; 36–165 for 5 GHz)

band:        str    # '2.4GHz' or '5GHz'

timestamp:   str    # ISO-8601 scan timestamp

## 17.4 Hardware Parameters

| Parameter | Typical Value | Impact on Design |
| --- | --- | --- |
| RSSI measurement range | −30 to −90 dBm | Defines normalisation bounds for signal sub-score |
| Supported bands | 2.4 GHz and 5 GHz | System must handle both frequency bands in parsed output |
| Scan completion time | 2–4 s (OS-controlled) | Sets minimum practical scan interval to 5 seconds |
| Maximum visible APs | Up to 50 in dense areas | Scoring and display must handle variable-length AP lists |
| RSSI measurement error | ±3–5 dBm (multipath fading) [5, 6] | Score uncertainty ≈ ±5–8 points; mitigated by 20-point switching threshold |

Table 20 – WiFi Adapter Hardware Parameters

# 18. Network Testing Subsystem – Detailed Design

## 18.1 Ping Command

The network testing subsystem measures the quality of the current active connection by sending ICMP Echo Request packets to the default network gateway. The gateway is identified programmatically using ipconfig and used as the ping target [7]. The exact command executed is:

import subprocess

result = subprocess.run(

['ping', '-n', '10', '-w', '2000', gateway_ip],

capture_output=True, text=True

)

| Parameter | Value | Justification |
| --- | --- | --- |
| -n 10 | 10 packets sent | Sufficient sample for statistically reliable packet loss (10% resolution); matches RFC 2544 [9] |
| -w 2000 | 2000 ms per-packet timeout | Allows for high-latency congested links; 2 s upper bound is practical for user experience assessment |
| Packet size | 32 bytes (default) | Small probe packets do not themselves cause additional congestion on the tested link |
| Target | Default gateway (ipconfig) | Closest network hop; measures only local AP-level congestion, not internet backbone latency |

Table 21 – Ping Parameters and Engineering Justification

## 18.2 Derived Performance Metrics

| Metric | Calculation | Unit |
| --- | --- | --- |
| Mean latency | Arithmetic mean of all received RTT values from 10-packet sample | ms |
| Minimum latency | Minimum RTT in the sample | ms |
| Maximum latency | Maximum RTT in the sample | ms |
| RTT variance | Var(RTT) = (1/n) × Σ(RTTᵢ − RTT̄)² where n = 10 | ms² |
| Jitter | Standard deviation of RTT sample; approximates RFC 3393 jitter | ms |
| Packet loss | (Packets sent − Packets received) / Packets sent × 100 | % |

Table 22 – Derived Network Performance Metrics

## 18.3 Stability Score Derivation

The Stability metric is derived from the RTT variance. High variance indicates an unstable connection where individual packets experience very different delays, characteristic of a congested AP where the queue length fluctuates [3]:

Var(RTT) = (1/n) × Σ(RTTᵢ − RTT̄)²    [n = 10 samples]

S_stability = clamp( 100 − (Var / 25), 0, 100 )

## 18.4 PingResult Data Structure

@dataclass

class PingResult:

target_ip:        str    # IP address of gateway or fallback DNS

mean_latency_ms:  float  # Mean RTT across 10 packets (ms)

min_latency_ms:   float  # Minimum RTT (ms)

max_latency_ms:   float  # Maximum RTT (ms)

packet_loss_pct:  float  # Percentage packets lost (0.0 – 100.0)

variance_ms2:     float  # RTT variance (ms²) – stability input

jitter_ms:        float  # Standard deviation of RTT (ms)

timestamp:        str    # ISO-8601 test timestamp

# 19. Intelligent Scoring Subsystem – Detailed Design

## 19.1 Normalisation Specifications

Each raw metric is linearly normalised to [0, 100] before applying the weights. The normalisation bounds are derived from IEEE 802.11 performance standards and established network quality benchmarks [4, 5].

### Signal Strength Normalisation

S_signal = clamp( (RSSI_dBm + 90) / 60 × 100,  0, 100 )

| RSSI (dBm) | S_signal | Classification |
| --- | --- | --- |
| −30 dBm | 100 | Excellent – very close to AP |
| −50 dBm | 66.7 | Good – typical classroom proximity |
| −65 dBm | 41.7 | Fair – usable but moderate attenuation |
| −75 dBm | 25.0 | Weak – near edge of reliable coverage [5] |
| −90 dBm | 0 | No signal – minimum threshold |

Table 23 – Signal Score Normalisation Examples

### Latency Normalisation

S_latency = clamp( (200 − mean_latency_ms) / 190 × 100,  0, 100 )

### Packet Loss Normalisation

S_loss = clamp( (20 − packet_loss_pct) / 20 × 100,  0, 100 )

### Stability Normalisation

S_stability = clamp( 100 − (variance_ms2 / 25),  0, 100 )

## 19.2 Recommendation Decision Logic

The system issues a recommendation only when ALL three conditions are simultaneously satisfied:

- The current AP score is below 75 (not GOOD)

- At least one other visible AP has a score greater than the current AP by at least 20 points

- The recommended AP is on a compatible band (5 GHz preferred over 2.4 GHz for equivalent scores)

The 20-point threshold is an engineering decision: it exceeds the combined measurement uncertainty of ±5–12 points identified in Section 21, preventing recommendations based on measurement noise rather than genuine quality differences.

## 19.3 ScoredAP Data Structure

@dataclass

class ScoredAP:

ap_record:      APRecord    # Full AP scan record from scanner.py

ping_result:    PingResult  # None if AP not currently connected

s_signal:       float       # Normalised signal sub-score  [0–100]

s_latency:      float       # Normalised latency sub-score [0–100]

s_loss:         float       # Normalised loss sub-score    [0–100]

s_stability:    float       # Normalised stability score   [0–100]

total_score:    float       # Weighted composite score     [0–100]

status:         str         # 'GOOD' | 'MODERATE' | 'CONGESTED'

is_connected:   bool        # True if this is the currently active AP

is_recommended: bool        # True if recommended by decision logic

# 20. Data Storage Subsystem – Detailed Design

## 20.1 CSV Schema

Each row in the CSV file represents one AP measurement at one point in time. The schema is defined as follows:

| Column Name | Data Type | Source | Example |
| --- | --- | --- | --- |
| timestamp | ISO-8601 datetime | System clock | 2026-06-10T14:23:05 |
| ssid | String | APRecord | Eduroam |
| bssid | String (MAC) | APRecord | AA:11:22:33:44:55 |
| channel | Integer | APRecord | 6 |
| band | String | APRecord | 2.4GHz |
| rssi_dbm | Float | APRecord | −62.0 |
| signal_pct | Integer | APRecord | 76 |
| mean_latency_ms | Float | PingResult | 87.3 |
| packet_loss_pct | Float | PingResult | 3.0 |
| variance_ms2 | Float | PingResult | 412.5 |
| s_signal | Float | ScoredAP | 46.7 |
| s_latency | Float | ScoredAP | 59.3 |
| s_loss | Float | ScoredAP | 85.0 |
| s_stability | Float | ScoredAP | 83.5 |
| total_score | Float | ScoredAP | 62.4 |
| status | String | ScoredAP | MODERATE |
| is_connected | Boolean (0/1) | ScoredAP | 1 |
| is_recommended | Boolean (0/1) | ScoredAP | 0 |

Table 24 – CSV File Schema (18 Columns)

## 20.2 File Naming and Write Operation

import csv, os

from datetime import datetime

LOG_DIR = './logs/'

os.makedirs(LOG_DIR, exist_ok=True)

# New file per session

filename = f"wifi_log_{datetime.now().strftime('%Y%m%d_%H%M%S')}.csv"

filepath = os.path.join(LOG_DIR, filename)

def write_records(scored_aps: list[ScoredAP], filepath: str):

file_exists = os.path.isfile(filepath)

with open(filepath, 'a', newline='', encoding='utf-8') as f:

writer = csv.DictWriter(f, fieldnames=SCHEMA_COLUMNS)

if not file_exists:

writer.writeheader()  # Write header only on first write

for ap in scored_aps:

writer.writerow(ap.to_dict())

## 20.3 Storage Capacity Calculation

- Monitoring cycle interval: 10 seconds

- Maximum APs per scan: 20 (typical dense indoor environment)

- Bytes per CSV row: approximately 200 bytes (18 fields × ~11 characters average)

- Rows per hour: (3600 / 10) × 20 = 7,200 rows/hour

- Storage per hour: 7,200 × 200 bytes ≈ 1.4 MB/hour

- Full 8-hour session: 1.4 × 8 = 11.2 MB

- Full semester (120 active hours): 1.4 × 120 ≈ 168 MB  →  negligible on any modern laptop

# 21. Engineering Calculations

## 21.1 Shannon-Hartley Per-User Throughput

The Shannon-Hartley Theorem defines the maximum channel capacity for a 2.4 GHz 802.11n channel of bandwidth B = 20 MHz at SNR = 20 dB (linear SNR = 100) [2]:

C_total = 20×10⁶ × log₂(1 + 100) = 20×10⁶ × 6.658 ≈ 133 Mbps

Accounting for 802.11n protocol overhead, CSMA/CA collisions, and guard intervals (efficiency ≈ 45%):

C_practical ≈ 133 × 0.45 ≈ 60 Mbps  (practical AP channel capacity)

| Users on AP | Throughput / User | User Experience |
| --- | --- | --- |
| 5 | 60 / 5 = 12.0 Mbps | Excellent – software downloads complete in seconds |
| 15 | 60 / 15 = 4.0 Mbps | Acceptable – noticeable slowdown during peak |
| 30 | 60 / 30 = 2.0 Mbps | Poor – large downloads take minutes |
| 60 | 60 / 60 = 1.0 Mbps | Severe congestion – system timeout risk |
| 80 | 60 / 80 = 0.75 Mbps | Critical – consistent failures expected |

Table 25 – Per-User Throughput vs AP Load (Shannon-Hartley Model) [2]

## 21.2 M/M/1 Queueing Latency Prediction

Using the M/M/1 model with AP service rate μ = 1000 packets/second and average user rate λ_user = 50 packets/second [3]:

| Users | λ_total (pkt/s) | μ − λ | W (ms) | Status |
| --- | --- | --- | --- | --- |
| 5 | 250 | 750 | 1.3 ms | GOOD |
| 10 | 500 | 500 | 2.0 ms | GOOD |
| 15 | 750 | 250 | 4.0 ms | MODERATE |
| 18 | 900 | 100 | 10.0 ms | MODERATE |
| 19 | 950 | 50 | 20.0 ms | CONGESTED |
| 20 | 1000 | → 0 | → ∞ | Saturation – queue collapse |

Table 26 – M/M/1 Model: Latency vs User Load [3]

This non-linear relationship justifies weighting latency at 30% – the second highest weight – in the scoring formula. Small increases in user load near saturation cause disproportionately large latency increases [3].

## 21.3 Measurement Accuracy Analysis

| Metric | Error Source | Magnitude | Score Impact |
| --- | --- | --- | --- |
| RSSI (dBm) | RF multipath fading; adapter calibration [5, 6] | ±3–5 dBm | ±5–8 score points |
| Latency (ms) | OS scheduling jitter; background processes | ±2–5 ms | ±1–3 score points |
| Packet loss | Statistical sampling error (n=10) | ±10% | ±5 score points |
| Variance | Small sample noise | ±100–200 ms² | ±4–8 score points |
| Total score | Combined weighted error propagation | ±5–12 points | Boundary misclassification risk |

Table 27 – Measurement Accuracy Analysis and Score Impact

The combined score uncertainty of ±5–12 points is the engineering justification for the 20-point switching threshold. A threshold below the measurement uncertainty would trigger recommendations based on noise rather than genuine quality differences.

## 21.4 CPU and Memory Footprint

| Operation | CPU Time | Memory |
| --- | --- | --- |
| netsh WiFi scan | 2–4 s (OS I/O wait) | < 1 MB |
| Text parsing (regex) | < 5 ms | < 1 MB |
| Ping test (10 × 2 s max) | 2–5 s (I/O wait) | < 1 MB |
| Scoring (20 APs) | < 1 ms | < 1 MB |
| CSV write (20 rows) | < 5 ms | < 1 MB |
| Streamlit dashboard | < 100 ms per render | ~30 MB (browser tab) |
| Total per cycle | ~4–10 s active | ~35 MB resident |

Table 28 – CPU and Memory Footprint Estimate

The 35 MB memory footprint is negligible on any laptop with 4+ GB RAM. The application will not perceptibly impact normal student laptop usage.

# 22. System Integration Design

## 22.1 Configuration Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| SCAN_INTERVAL | 10 s | Time between complete monitoring cycles |
| PING_COUNT | 10 | ICMP packets per ping test |
| PING_TIMEOUT_MS | 2000 | Per-packet timeout (ms) |
| RSSI_MIN / MAX | −90 / −30 | Normalisation bounds for signal sub-score |
| LATENCY_MIN / MAX | 10 / 200 ms | Normalisation bounds for latency sub-score |
| LOSS_MAX | 20% | Upper bound for packet loss normalisation |
| VARIANCE_MAX | 2500 ms² | Upper bound for stability normalisation |
| W_SIGNAL | 0.35 | Scoring weight – signal strength |
| W_LATENCY | 0.30 | Scoring weight – latency |
| W_LOSS | 0.20 | Scoring weight – packet loss |
| W_STABILITY | 0.15 | Scoring weight – stability |
| THRESHOLD_GOOD | 75 | Score above which AP classified GOOD |
| THRESHOLD_MOD | 50 | Score above which AP classified MODERATE |
| SWITCH_THRESHOLD | 20 | Min score improvement to trigger recommendation |
| LOG_DIR | ./logs/ | Directory for CSV log files |
| PING_FALLBACK | 8.8.8.8 | DNS fallback if gateway discovery fails |

Table 29 – config.py Parameters with Defaults

## 22.2 End-to-End Worked Example

The following worked example traces one complete monitoring cycle using realistic values from a congested university laboratory environment.

Figure 12 – Worked Example: Score decomposition for three APs showing CONGESTED classification with no recommendation issued

### Input Measurements

| AP | BSSID (short) | Signal% | RSSI (dBm) | Connected? |
| --- | --- | --- | --- | --- |
| AP-01 | AA:11:22 | 60% | −70.0 dBm | YES |
| AP-02 | BB:33:44 | 80% | −60.0 dBm | NO |
| AP-03 | CC:55:66 | 40% | −80.0 dBm | NO |

Ping result for connected AP-01: mean latency = 145 ms, packet loss = 6%, variance = 1800 ms²

### Step 1: Normalisation for AP-01 (connected)

- S_signal   = (−70 + 90) / 60 × 100  =  20/60 × 100  =  33.3

- S_latency  = (200 − 145) / 190 × 100 =  55/190 × 100  =  28.9

- S_loss     = (20 − 6) / 20 × 100     =  14/20 × 100   =  70.0

- S_stability = 100 − (1800 / 25)       =  100 − 72      =  28.0

### Step 2: Score Calculation for AP-01

Score = 0.35×33.3 + 0.30×28.9 + 0.20×70.0 + 0.15×28.0 = 11.7+8.7+14.0+4.2 = 38.6  →  CONGESTED

### Step 3: AP-02 and AP-03 (not connected – signal score only, other sub-scores default to 50)

Score_AP02 = 0.35×50.0 + 0.30×50.0 + 0.20×50.0 + 0.15×50.0 = 50.0  →  MODERATE

Score_AP03 = 0.35×16.7 + 0.30×50 + 0.20×50 + 0.15×50 = 38.3  →  CONGESTED

### Step 4: Recommendation Decision

- Current AP (AP-01) score: 38.6 – below GOOD threshold (75)  ✓

- Best alternative: AP-02, score 50.0  |  Delta = 50.0 − 38.6 = 11.4

- Delta (11.4) is BELOW switching threshold (20)  ✗

- Result: No recommendation issued. Dashboard message: 'Current AP congested. No significantly better AP detected. Consider moving to a different area.'

# 23. User Interface Subsystem – Detailed Design

## 23.1 Implementation: Streamlit

The dashboard is implemented using Streamlit, a Python framework that renders interactive web dashboards directly from Python code accessible via the browser at http://localhost:8501. No HTML, CSS, or JavaScript is required. Streamlit is installed via pip without administrator rights and is the chosen framework because it provides native support for real-time refresh (st.rerun()), colour-coded metric boxes (st.metric), and inline chart rendering (st.line_chart, st.bar_chart) [10].

## 23.2 Streamlit Component Mapping

| Dashboard Component | Streamlit Element | Content |
| --- | --- | --- |
| Page header | st.title + st.caption | System name, scan count, last updated timestamp |
| Current AP status | st.metric (3 columns) | AP name, score, status badge |
| Status alert | st.success / st.warning / st.error | Colour-coded GOOD / MODERATE / CONGESTED alert |
| AP table | st.dataframe with Styler | All APs: BSSID, RSSI, latency, loss, score, status |
| Recommendation | st.info or st.warning block | Recommended AP name, score delta, directional advice |
| Signal bar chart | st.bar_chart | RSSI values for all detected APs |
| Latency trend | st.line_chart | 30-second rolling latency window for connected AP |
| Raw data expander | st.expander + st.dataframe | Full PingResult and APRecord on demand |

Table 30 – Streamlit Component Mapping

## 23.3 Recommendation Message Templates

| Condition | Message Displayed |
| --- | --- |
| Current AP is GOOD | Connected to best available AP. Score: {score}/100. No action needed. |
| Better AP available (ΔScore ≥ 20) | Better connection available: {ssid} ({bssid}) – Score: {alt_score}/100. Move toward that area to improve your speed. Improvement: +{delta} points. |
| Congested, no better alternative | Current AP is congested (Score: {score}/100). No significantly better AP detected. Consider moving to a different area of the building. |
| Scan returned no APs | No WiFi networks detected. Please verify that WiFi is enabled on this device. |

Table 31 – Recommendation Message Templates

# 24. Unit and Integration Testing Plan

## 24.1 Unit Tests

| Test ID | Module | Description | Pass Criterion |
| --- | --- | --- | --- |
| UT-01 | scanner.py | Execute netsh scan; verify ≥ 1 APRecord with valid BSSID returned | List length ≥ 1; BSSID matches MAC regex ^([0-9A-Fa-f]{2}:){5}[0-9A-Fa-f]{2}$ |
| UT-02 | scanner.py | Inject Signal% = 72; verify derived RSSI = −64.0 dBm | RSSI within ±0.1 dBm of −64.0 |
| UT-03 | tester.py | Execute ping to 8.8.8.8 n=10; verify PingResult populated | mean_latency_ms > 0; packet_loss_pct in [0, 100] |
| UT-04 | scorer.py | Input RSSI=−45, latency=30 ms, loss=0%, variance=0; verify score ≥ 75 | total_score ≥ 75; status = 'GOOD' |
| UT-05 | scorer.py | Input RSSI=−88, latency=195 ms, loss=18%, variance=2400; verify score < 50 | total_score < 50; status = 'CONGESTED' |
| UT-06 | scorer.py | Two APs: current score=40, alternative score=65 (delta=25); verify recommendation fires | is_recommended = True on alt AP |
| UT-07 | scorer.py | Two APs: current score=40, alternative score=59 (delta=19); verify no recommendation | is_recommended = False on all APs |
| UT-08 | storage.py | One monitoring cycle; verify CSV created with correct 18-column header and rows | Header matches Table 24 schema; row count = AP count |
| UT-09 | storage.py | Verify timestamp field parses as valid ISO-8601 | datetime.fromisoformat(timestamp) succeeds without exception |

Table 32 – Unit Test Specifications with Pass Criteria

## 24.2 Integration Tests

| Test ID | Scenario | Method | Pass Criterion |
| --- | --- | --- | --- |
| IT-01 | Low density: 1–3 devices | Monitor during off-peak hours; 10 cycles | Score ≥ 75 in ≥ 8 of 10 cycles; no recommendation |
| IT-02 | High density: 15+ devices | Monitor during lab session; 10 cycles | Score < 50 detected; CONGESTED shown; recommendation fires |
| IT-03 | Physical movement test | Move laptop across room; record score per cycle | Score changes ≥ 10 points over 3+ cycles; trend visible in CSV |
| IT-04 | Multi-AP comparison | Scan in area with ≥ 3 visible APs | All APs shown in table with distinct scores; best AP highlighted |
| IT-05 | 30-minute continuous operation | Leave system running; verify no crashes | No unhandled exceptions; CSV grows at expected rate; refresh every 10 s |

Table 33 – Integration Test Specifications

# 25. Known Limitations and Future Work

## 25.1 Known Limitations

| Limitation | Engineering Root Cause | System Impact |
| --- | --- | --- |
| Ping data available for connected AP only | Windows ICMP tests not possible through non-connected interfaces | Unconnected APs scored on signal alone; ranking uncertainty introduced |
| RSSI noise of ±3–5 dBm [5, 6] | RF multipath fading and adapter calibration variation | Score uncertainty ±8 points; mitigated by 20-point threshold |
| netsh scan takes 2–4 s (OS-controlled) | Windows WLAN service controls scan timing internally | Minimum cycle is 5–10 s; real-time reaction to sudden congestion limited |
| Hidden AP load data unavailable | Client count not broadcast in 802.11 beacon frames [4] | Congestion inferred indirectly from latency/loss; no direct user count |
| Windows-only prototype | netsh is Windows-specific [7] | Cross-platform port requires iwlist (Linux) or airport (macOS) |

Table 34 – Known Limitations and Root Causes

## 25.2 Future Work

- Machine learning scoring model: replace fixed weights with a Random Forest model trained on labelled AP quality data collected during testing.

- Android mobile prototype: implement sensing using Android WifiManager API to extend to smartphones and tablets.

- Multi-node architecture: deploy Raspberry Pi sensing nodes in multiple rooms, aggregating data to a central dashboard via MQTT protocol.

- Automatic AP switching: investigate 802.11r BSS Fast Transition support on campus APs to enable seamless roaming triggered by the recommendation engine [4].

- Live user count estimation: correlate latency patterns with user density using M/M/1 model inversion to estimate connected users per AP [3].

# References

[1]  Engineering Council of South Africa (ECSA), Accreditation Standard for Engineering Programmes, Pretoria: ECSA, 2019.

[2]  C. E. Shannon, "A Mathematical Theory of Communication," Bell System Technical Journal, vol. 27, no. 3, pp. 379–423, July 1948.

[3]  L. Kleinrock, Queueing Systems Volume 1: Theory. New York: John Wiley & Sons, 1975.

[4]  IEEE 802.11-2020, IEEE Standard for Information Technology – Telecommunications and Information Exchange Between Systems – Local and Metropolitan Area Networks – Specific Requirements – Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications. New York: IEEE, 2020.

[5]  A. Goldsmith, Wireless Communications. Cambridge: Cambridge University Press, 2005.

[6]  D. Tse and P. Viswanath, Fundamentals of Wireless Communication. Cambridge: Cambridge University Press, 2005.

[7]  Microsoft Corporation, "NDIS_WLAN_ASSOCIATION_ATTRIBUTES structure (windot11.h)," Windows Hardware Developer Documentation, 2023. [Online]. Available: https://docs.microsoft.com/en-us/windows-hardware/drivers/ddi/windot11/

[8]  V. Jacobson, "Congestion avoidance and control," in Proc. ACM SIGCOMM, Stanford, CA, 1988, pp. 314–329.

[9]  RFC 2544, "Benchmarking Methodology for Network Interconnect Devices," Internet Engineering Task Force (IETF), Feb. 1999.

[10] Streamlit Inc., Streamlit Documentation, Version 1.32, 2024. [Online]. Available: https://docs.streamlit.io